Transport Mechanism Selection for Cisco Secure Client Remote Access VPN

Enterprise Deployment – Windows 11

Author: Miroslav Ivanov

Date: 27.01.26

Contents

1.  Purpose and Scope	2

2. Transport Options Considered	2

3.  Key Trade-offs and Comparative Analysis	3

4.  Decision and Recommendation	5

# Purpose and Scope

# This document evaluates Remote Access VPN transport options for Cisco Secure Client in an enterprise deployment comprising approximately 2,000–3,000 Windows 11 endpoints.

# The scope of this decision is limited to selection of the VPN data-plane transport for remote access connectivity. Cisco Secure Client supports multiple VPN transport mechanisms; however, this analysis considers only the following enterprise-relevant options:

# - SSL-based VPN (TLS/DTLS)

# - IPsec-based VPN (IKEv2)

# Cisco Identity Services Engine (ISE) integration for authentication, endpoint posture assessment, and policy enforcement is treated as a mandatory architectural requirement.

The selected transport MUST provide reliable remote access connectivity under heterogeneous and restrictive network conditions, preserve enterprise security posture, exhibit predictable performance characteristics at scale and minimize operational overhead related to deployment, support, and troubleshooting.

# 2. Transport Options Considered

The analysis examines the characteristics, trade-offs, and operational behavior of the selected transport options.

## 2.1 SSL-Based Transport  (TLS  /  DTLS)

Cisco Secure Client supports SSL-based VPN connectivity using:

- TLS over TCP 443

- DTLS over UDP 443

TLS over TCP 443 ensures maximum compatibility across enterprise, residential, and public networks, as TCP 443 is almost universally permitted through firewalls, NAT devices, and proxy-controlled environments. As a result, TLS serves as a highly reliable transport for establishing VPN connectivity in restrictive or unpredictable network conditions.

DTLS operates over UDP 443 and is optimized for VPN data-plane traffic. By avoiding TCP congestion control and retransmission behavior at the transport layer, DTLS reduces head-of-line blocking and improves performance under packet loss, variable latency, and high-throughput conditions.

Cisco Secure Client establishes the VPN control channel over TLS and opportunistically negotiates DTLS for VPN data traffic when UDP connectivity is available. In environments where DTLS cannot be negotiated, the client reverts to TLS-based data transport without user interaction.

## 2.2 IPsec-Based Transport (IKEv2)

Cisco Secure Client also supports IPsec-based Remote Access VPN using the IKEv2 protocol. In a typical deployment, this transport relies on the following mechanisms:

- UDP 500 and UDP 4500 for IKEv2 negotiation and NAT traversal

- Encapsulating Security Payload (ESP) traffic for data transport

IPsec/IKEv2 provides a mature and well-defined security model with efficient packet processing at the network layer. However, its dependence on specific UDP ports and ESP encapsulation can reduce connection success rates in restrictive or tightly controlled network environments, particularly those enforcing strict firewall policies, protocol filtering, or carrier-grade NAT.

While IPsec-based Remote Access VPN is fully supported by Cisco Secure Client and integrates with Cisco Identity Services Engine (ISE) for authentication and policy enforcement, its operational characteristics differ materially from SSL-based VPN transports. These differences are evaluated further in the comparison section.

# 3. Key Trade-offs and Comparative Analysis

This section compares SSL-based (TLS/DTLS) and IPsec-based (IKEv2) Remote Access VPN transports across key dimensions that materially impact connectivity reliability, performance, and operational supportability in large-scale enterprise deployments.

### 3.1 Key Comparison Dimensions

Network Traversal & NAT Compatibility

- TLS/DTLS: NAT-friendly, uses TCP/UDP 443

- IKEv2: NAT-T required, sensitive to UDP state and middlebox behavior

Firewall & Proxy Compatibility

- TLS/DTLS: Commonly permitted, proxy-aware

- IKEv2: Requires explicit exceptions (UDP 500/4500, ESP)

Connection Resiliency

- TLS/DTLS: Opportunistic DTLS with automatic TLS fallback at establishment; runtime fallback requires DPD

- IKEv2: Recovery often requires tunnel re-establishment

Performance

- TLS: Acceptable baseline performance

- DTLS: High performance, avoids TCP-over-TCP

- IKEv2: Efficient when permitted

Operational Complexity

- TLS/DTLS: Minimal tuning, predictable behavior

- IKEv2: Higher troubleshooting complexity

## 3.2 Summary Comparison

| Dimension | TLS / DTLS | IPsec (IKEv2) |
| --- | --- | --- |
| NAT compatibility | High | Medium |
| Firewall traversal | High | Medium to Low |
| Connection success rate | Very High | Variable |
| Performance (optimal conditions) | High (DTLS) | High |
| Performance (lossy networks) | Medium (TLS) / High (DTLS) | Medium |
| Failure recovery | Graceful | Reconnect-oriented |
| Operational complexity | Low | Medium to High |
| Suitability for heterogeneous networks | High | Medium |

## 3.3 Comparative Assessment

While both transports provide strong cryptographic security and are supported by Cisco Secure Client, SSL-based VPN transports deliver superior resilience, predictability, and operational simplicity across heterogeneous enterprise environments. IPsec-based transports remain viable in controlled networks but introduce increased risk of connectivity and support challenges at scale.

# 4. Decision and Recommendation

Cisco design guidance emphasizes transport flexibility, deterministic fallback behavior, and predictable operation across heterogeneous remote access environments. Based on Cisco product documentation, design guides, and operational best practices, the following transport selection is recommended for this deployment.

4.1 Decision

The selected Remote Access VPN transport for this deployment is SSL-based VPN using TLS and DTLS.

- TLS over TCP 443 SHALL be used for VPN session establishment and as the deterministic data-plane fallback.

- DTLS over UDP 443 SHALL be enabled and used as the preferred data-plane transport when UDP connectivity is available.

- Dead Peer Detection (DPD) SHALL be enabled to support seamless runtime fallback     from DTLS to TLS without user intervention or full tunnel re-establishment.

This transport model ensures predictable connectivity and operational stability across heterogeneous and restrictive remote network environments.

## 4.2 Rationale

The transport selection conforms to Cisco design guidance and best practices for large-scale Remote Access VPN deployments.

SSL-based VPN transports enable:

- High network compatibility through consistent traversal of firewalls, NAT devices, and proxy-controlled environments using TCP/443 and UDP/443.

- Deterministic fallback behavior, ensuring VPN availability under changing or restrictive network conditions.

- Optimized performance, leveraging DTLS where available while retaining TLS as a universal fallback.

- Lower operational complexity, reducing environment-specific tuning and support overhead at scale.

## 4.3 Alignment with Cisco Identity Services Engine (ISE)

Predictable transport behavior is required to ensure stable authentication, posture assessment, and authorization workflows with Cisco Identity Services Engine (ISE).

SSL-based transports minimize variability caused by NAT behavior, UDP state expiration, and mid-session renegotiation, improving the reliability of posture evaluation and policy enforcement.

## 4.4 Non-Selected Option

IPsec-based Remote Access VPN using IKEv2 is not selected as the default transport.

While supported by Cisco Secure Client and ISE, IPsec/IKEv2 introduces greater dependency on specific UDP ports and ESP encapsulation, resulting in reduced predictability and increased operational risk in heterogeneous remote access environments.

This document concludes that SSL-based Remote Access VPN using TLS/DTLS provides the most predictable, scalable, and operationally stable transport for Cisco Secure Client in large-scale Windows 11 enterprise deployments.