Hardened Enterprise Edge – Layer 2 Security Architecture and Control Validation

Author: Miroslav Ivanov

Date: 20.02.26

Contents

1. Executive Summary	2

2. Threat Model Assumptions	3

3. Foundational Defensive Posture Establishment	3

3.1 Architecture overview	3

3.2 Attack Surface Reduction	4

3.3 Management Plane Isolation	4

4. DHCP Snooping – Trust Boundary Model	5

4.1 Objective	5

4.2 Functional Overview	5

4.3 Threat Exposure and Mitigation	6

4.4 Observed Protocol Behavior – DHCP Option 82 Handling	6

4.5 Rogue DHCP Server Validation	8

5. DHCP Starvation Mitigation and Identity Enforcement	10

5.1 Rate Limiting Configuration	10

5.2 Starvation Attack Testing	11

6. ARP Validation and Integrity Enforcement	14

6.1 ARP Protocol Vulnerability	14

6.2 Observed ARP Inconsistency Simulation	14

6.3 DAI Dependency on DHCP Snooping	15

7. Layered Defense Model – Enterprise Edge Validation Results	17

# 1. Executive Summary

This document describes the design, implementation, validation, and behavioral analysis of Layer 2 security controls within a segmented enterprise edge laboratory environment.

The objective of this lab was to:

- Implement defensive Layer 2 trust enforcement mechanisms.

- Harden unused interfaces.

- Enforce management plane isolation.

- Validate DHCP & ARP related attack protections.

- Analyze the roles and limitations of individual defense mechanisms within a layered Layer 2 security model.

- Evaluate tool limitations (Packet Tracer).

## 2. Threat Model Assumptions

This laboratory environment assumes the following adversarial capabilities:

- Attacker has physical access to access ports

- Attacker may deploy rogue DHCP server

- Attacker may perform ARP spoofing

- Attacker may rotate MAC identities

- Attacker has no direct infrastructure credentials

The objective of the implemented controls is to prevent unauthorized control-plane manipulation and data-plane integrity compromise within shared broadcast domains under these constraints.

# 3. Foundational Defensive Posture Establishment

## 3.1 Architecture overview

The lab topology consisted of:

- Segmented VLAN architecture:

- Access Layer Switches (SW1, SW2)

- Distribution Switch (SW3)

- Edge Router (R1)

For ease of configuration and controlled validation, the DHCP server was implemented directly on the edge router (R1).

Before implementing active Layer 2 trust enforcement mechanisms, the first step was to reduce the exposed attack surface and establish clear logical segmentation boundaries. Security controls are most effective when applied on top of a controlled and segmented baseline. Therefore, foundational hardening was performed prior to attack validation.

Routing was performed via Router-on-a-Stick with OSPF for internal route advertisement.

All VLANs were interconnected at Layer 3; therefore, Layer 2 trust enforcement mechanisms were critical to prevent intra-domain abuse and control-plane manipulation.

| VLAN | Purpose |
| --- | --- |
| 10 | Dev |
| 20 | Sales |
| 30 | Management |
| 40 | Misc / Black-hole |
| 50 | Native |

Establish deterministic broadcast domains and prevent uncontrolled lateral movement.

Segmentation alone does not provide security; however, it provides:

- Controlled broadcast domains

- Defined traffic boundaries

- Predictable enforcement zones

## 3.2 Attack Surface Reduction

Minimize exposed Layer 2 ingress points.

All unused interfaces were:

- Assigned to VLAN 40 (Black-hole VLAN)

- Administratively shut down

## 3.3 Management Plane Isolation

Separate control-plane access from user data-plane traffic.

### 3.3.1 Implementation

- Local administrative account configured.

- SSH enabled (Telnet disabled).

- login local applied to:

  - VTY lines 0–4

  - Console 0

- VLAN 30 SVI created for management.

- ACL applied restricting access to the management subnet only.

### 3.3.2 Security Risks Addressed

Without management isolation:

- Any user VLAN could attempt SSH access.

- Lateral privilege escalation becomes possible.

- Control plane shares trust level with data plane.

To validate baseline functionality prior to security enforcement, few endpoint devices were deployed:

- PC0 connected to SW1 Fa0/2 in VLAN 10 (Dev)

- PC1 connected to SW1 in VLAN 20 (Sales)

- Management Server to SW1 in VLAN 30 (Mngt)

- PC2 connected to SW2 Fa0/3 in VLAN 10

- Rogue Server to SW2 Fa0/1 in unmanaged VLAN 1 for additional penetration testing

With this configuration:

- Inter-VLAN routing via Router-on-a-Stick was verified.

- DHCP services provided by the edge router were confirmed operational.

- Management plane isolation through VLAN 30 and ACL enforcement was validated.

At this stage, the network was functionally operational but not yet hardened against Layer 2 control-plane abuse. With baseline connectivity confirmed, the next phase focused on activating DHCP Snooping across all Layer 2 devices to establish explicit trust boundaries and prevent rogue DHCP behavior.

# 4. DHCP Snooping – Trust Boundary Model

## 4.1 Objective

The objective of enabling DHCP Snooping was to establish an explicit Layer 2 trust boundary between infrastructure-facing interfaces and user-facing access ports, and to prevent rogue DHCP-related attacks(DHCP Spoofing and Starvation) within shared broadcast domains.

DHCP Snooping was enabled globally and enforced on VLANs 10, 20, 30, 40, and 50.

This ensured that DHCP control-plane traffic was subject to validation across all operational VLANs.

## 4.2 Functional Overview

When DHCP Snooping is enabled and enforced on specified VLANs, the following behavior is applied:

- All switch interfaces are classified as untrusted by default.

- DHCP server control-plane messages (DHCP OFFER, DHCP ACK) received on untrusted interfaces are automatically dropped.

- Only interfaces explicitly configured as trusted are permitted to forward DHCP server responses.

- DHCP client-originated messages (DHCP DISCOVER, DHCP REQUEST) are allowed on untrusted access interfaces.

- A DHCP Snooping binding table is dynamically constructed upon successful lease assignment.

The binding table records the following attributes:

- MAC Address

- Assigned IP Address

- VLAN ID

- Ingress Interface

This binding table serves as the authoritative source for subsequent Layer 2 validation mechanisms such as Dynamic ARP Inspection (DAI) and IP Source Guard.

## 4.3 Threat Exposure and Mitigation

### 4.3.1 Pre-Control Exposure

In the absence of DHCP Snooping, the switching domain remains vulnerable to the following attack vectors:

- Rogue DHCP server deployment

- Malicious default gateway distribution

- DNS redirection

- Man-in-the-middle (MITM) interception

- DHCP-based denial of service

Because DHCP operates within broadcast domains, any endpoint device can attempt to impersonate a legitimate server unless explicitly restricted.

### 4.3.2 Control Impact After Enforcement

With DHCP Snooping deployed:

• Infrastructure trust boundaries are explicitly defined.
• Only interfaces marked as trusted are permitted to forward DHCP server responses.
• Unauthorized DHCP server responses are prevented from propagating within the broadcast domain.
• Lease integrity is preserved through binding table validation.

This significantly reduces the feasibility of DHCP spoofing within the broadcast domain.

### 4.3.3 Residual Risk

While DHCP Snooping prevents rogue server behavior, it does not fully eliminate DHCP starvation attacks caused by rapid MAC identity rotation.

## 4.4 Observed Protocol Behavior – DHCP Option 82 Handling

### 4.4.1 Observed Behavior

After enabling DHCP Snooping across all Layer 2 devices, the DHCP server ceased distributing IP addresses to endpoint devices.

Clients were unable to obtain leases despite correct VLAN configuration and trusted uplink designation.

Debug output indicated that DHCP packets were being dropped due to the presence of a non-zero giaddr field or DHCP Option 82 information received on untrusted interfaces.

### 4.4.2 Root Cause Analysis

When DHCP Snooping is enabled, switches may automatically insert DHCP Option 82 (Relay Agent Information Option) into DHCP DISCOVER and REQUEST packets received from clients.

Option 82 adds metadata identifying:

- The ingress interface (Circuit ID)

- The relay device identity (Remote ID)

Additionally, DHCP Snooping enforces validation rules:

- If a DHCP packet containing a non-zero giaddr (Gateway IP Address field) or Option 82 information is received on an untrusted interface, the packet is dropped.

- This behavior prevents malicious relay injection or control-plane spoofing.

In this lab topology, multiple Layer 2 devices began inserting Option 82 information into forwarded DHCP packets. When those packets reached upstream switches that were configured to disallow Option 82 on untrusted ports, they were dropped.

This resulted in complete DHCP failure.

### 4.4.3 Protocol Mechanism Explanation

DHCP Option 82 is defined to support environments utilizing DHCP Relay Agents.

Its purpose is to:

- Identify the originating client access port.

- Provide contextual information to centralized DHCP servers.

- Enable policy-based address assignment.

- Improve traceability and subscriber accountability in large networks.

The giaddr field is traditionally populated by Layer 3 DHCP relay agents when forwarding DHCP broadcasts between subnets.

Switch-level insertion of Option 82 enhances visibility and policy enforcement in enterprise and service provider environments.

### 4.4.4 Resolution Applied in Lab Environment

In this laboratory environment:

- DHCP services were hosted directly on the edge router.

- No external relay architecture was required.

- Option 82 information was not needed.

To restore DHCP functionality, Option 82 insertion was disabled on all Layer 2 devices:

no ip dhcp snooping information option

This prevented switches from modifying DHCP packets and eliminated upstream drop conditions.

### 4.4.5 Architectural Insight

This behavior revealed an important architectural dependency:

Protocol behavior must be evaluated in the context of topology and trust boundaries.

Therefore, for Option 82-based designs to function correctly:

- All interfaces leading toward the legitimate DHCP server must be configured as trusted.

- All interfaces leading toward DHCP relay agents must also be configured as trusted.

- Trust configuration must align with relay insertion points.

Failure to align trust configuration with relay insertion points results in control-plane disruption and DHCP failure.

This lab demonstrated that:

- DHCP Snooping enforcement is sensitive to Option 82 handling.

- Trust configuration must reflect actual relay architecture.

- Enterprise-scale relay features introduce additional validation complexity.

- In smaller lab environments without centralized relay design, Option 82 insertion may be unnecessary and should be disabled.

Understanding the interaction between DHCP Snooping, relay agents, and Option 82 validation is essential when deploying Layer 2 trust enforcement in multi-switch enterprise environments.

## 4.5 Rogue DHCP Server Validation

### 4.5.1 Test Objective

The objective of this test was to validate that DHCP Snooping correctly prevents rogue DHCP server deployment within user VLANs.

The test aimed to confirm that:

- Unauthorized DHCP OFFER and ACK messages are dropped.

- Only explicitly trusted interfaces are permitted to distribute leases.

- Trust boundary enforcement operates as designed.

### 4.5.2 Attack Methodology

A rogue DHCP server was introduced into VLAN 10 and VLAN 20 via access interfaces on both the access layer and distribution layer switches to validate enforcement consistency across multiple switching tiers.

The following test scenarios were performed:

- Rogue server connected to an untrusted access port.

- DHCP service enabled on rogue device.

- Endpoint clients forced to renew DHCP leases.

- Observation of DHCP behavior via debug and binding table inspection.

A secondary validation scenario was conducted:

- The rogue server interface was temporarily configured as trusted to observe behavioral change.

### 4.5.3 Observed Behavior

When connected to an untrusted port:

- DHCP OFFER and ACK messages generated by the rogue server were dropped at ingress.

- Clients continued receiving valid leases from the legitimate DHCP server.

- No rogue bindings were created in the DHCP Snooping table.

When the rogue server interface was temporarily marked as trusted:

- Rogue DHCP OFFER messages were forwarded.

- Clients began receiving leases from the rogue server.

- DHCP control-plane integrity was compromised.

### 4.5.4 Control Validation

This test confirmed that:

- DHCP Snooping correctly enforces server-side trust boundaries.

- The trust flag on an interface directly determines DHCP authority.

- Rogue DHCP deployment is prevented when proper trust configuration is applied.

The attack was fully mitigated under correct trust configuration.

### 4.5.5 Architectural Insight

This validation reinforced a critical design principle:

The security boundary is defined by explicit trust configuration.

A single misconfigured trusted port immediately grants DHCP server authority within the broadcast domain.

Therefore:

- Infrastructure-facing ports must be carefully identified.

- Trust assignments must be minimal and deliberate.

- Periodic validation is required to ensure configuration integrity.

This test demonstrated that DHCP Snooping effectively protects against rogue DHCP spoofing when properly configured.

### 4.5.6 Tooling Limitation – Packet Tracer Observability

During adversarial testing, Packet Tracer provided limited visibility into DHCP Snooping drop events.

Debug output primarily displayed successful DHCP transactions, while dropped packets and detailed drop statistics were not consistently observable.

As a result, validation relied on endpoint lease verification and DHCP binding table inspection rather than direct drop logging.

This limitation indicates that Packet Tracer supports functional validation but lacks full control-plane observability compared to real IOS environments.

# 5. DHCP Starvation Mitigation and Identity Enforcement

## 5.1 Rate Limiting Configuration

To mitigate potential DHCP starvation attacks, rate limiting was applied to all access interfaces using the following configuration:

interface range fa0/1 - 24
ip dhcp snooping limit rate 15

This configuration restricts the number of DHCP packets permitted per second on each interface.

Rate limiting applies exclusively to DHCP protocol traffic and does not impact:

- ICMP

- TCP/UDP

- ARP

- OSPF

- General data-plane traffic

It is considered best practice to apply DHCP rate limiting only on user-facing access ports. Trunk interfaces and infrastructure-facing uplinks should not be rate-limited, as they may legitimately forward aggregated DHCP traffic from multiple downstream devices.

The purpose of this control is to reduce the velocity of DHCP starvation attempts. However, rate limiting does not prevent MAC address rotation or sustained pool exhaustion under determined attack conditions.

## 5.2 Starvation Attack Testing

### 5.2.1 Test Objective

The objective of this test was to evaluate the effectiveness and behavioral response of DHCP Snooping rate limiting under a DHCP starvation scenario.

The test aimed to determine:

- Whether DHCP rate limiting mitigates pool exhaustion

- Under what conditions interface err-disable is triggered

- The operational difference between mitigation and containment

### 5.2.2 Test Environment Preparation (Controlled Replication)

To simplify validation and accelerate observable behavior:

- The DHCP address pool for VLAN 10 was reduced to a small allocation range (192.168.10.10–192.168.10.15).

- A laptop acting as a rogue client was connected to SW2 on interface FastEthernet0/4 (VLAN 10).

- DHCP Snooping rate limit on Fa0/4 was intentionally lowered with:

interface fa0/4
ip dhcp snooping limit rate 2

The reduced threshold allowed rapid triggering of enforcement behavior for demonstration purposes.

### 5.2.3 Baseline Behavior – Single Lease Acquisition

A single DHCP renewal attempt was performed from the rogue client.

Observed behavior:

- DHCP REQUEST processed successfully

- Binding entry created

- Lease assigned from reduced pool

This confirmed normal DHCP operation under threshold limits.

### 5.2.4 Escalated Behavior – Rapid Renewal / MAC Rotation

The rogue client performed repeated DHCP renewals in rapid succession.

The interface was automatically placed into err-disable state after exceeding the configured rate threshold.

### 5.2.5 Behavioral Analysis

This test demonstrated that DHCP rate limiting exhibits two operational modes:

- Mitigation Mode
When configured with moderate thresholds, excessive DHCP packet generation is throttled but the interface remains operational.

- Containment Mode
When configured with aggressive thresholds, rate violations trigger err-disable, effectively quarantining the attacking interface.

However, DHCP rate limiting is packet-velocity-based, not identity-based.
It does not prevent MAC identity rotation under low-rate attack patterns.

Therefore, rate limiting mitigates velocity-based starvation.

### 5.2.6 Layered Security Model – Identity Enforcement

While DHCP rate limiting restricts packet velocity, it does not prevent a malicious endpoint from presenting multiple Layer 2 identities at controlled rates.

To address identity-based starvation vectors, Port Security was implemented on user-facing access interfaces:

switchport port-security
switchport port-security maximum 1
switchport port-security mac-address sticky
switchport port-security violation restrict

Port Security enforces:

- A maximum number of MAC addresses per interface

- Persistent MAC binding through sticky learning

- Violation handling upon identity change

When combined with DHCP Snooping rate limiting:

- Rate limiting mitigates packet velocity

- Port Security enforces identity constraints

- DHCP Snooping protects server trust boundaries

Together, these controls form a layered Layer 2 defense model that addresses both velocity-based and identity-based attack vectors.

The violation counter increment confirms enforcement of per-port identity constraints under MAC rotation attempts.

With DHCP server trust boundaries, starvation mitigation, and identity enforcement validated, address allocation integrity was established.
However, once an IP address is legitimately assigned, Layer 2 communication depends on ARP resolution, which operates independently of DHCP.

Unlike DHCP, ARP provides no inherent authentication or validation mechanism.
Therefore, the next phase of the laboratory focused on evaluating ARP-based attack vectors and enforcing data-plane integrity using Dynamic ARP Inspection (DAI) and IP Source Guard.

# ARP Validation and Integrity Enforcement

## 6.1 ARP Protocol Vulnerability

Unlike DHCP, ARP provides no built-in trust enforcement or authentication mechanism. Any endpoint within the broadcast domain may transmit unsolicited ARP replies claiming ownership of an arbitrary IP address. As a result, ARP poisoning and gateway impersonation remain viable even when DHCP Snooping is properly enforced.

## 6.2 Observed ARP Inconsistency Simulation

During controlled testing, duplicate IP addressing was introduced by assigning the default gateway IP address (192.168.10.1) to both the legitimate gateway interface and a rogue endpoint. This produced inconsistent ARP resolution behavior on the victim host, where 192.168.10.1 alternated between two different MAC addresses.

To reliably reproduce the condition in Packet Tracer, the victim ARP cache was cleared and ARP learning was triggered by generating traffic from the rogue endpoint. Additionally, the legitimate gateway interface for VLAN 10 was temporarily administratively disabled to allow the rogue endpoint to respond to ARP requests without contention. Under these conditions, the victim learned an incorrect Layer 2 next-hop for 192.168.10.1, resulting in loss of Layer 3 forwarding (traffic destined to the gateway was sent to the rogue MAC).

Note: When the legitimate gateway interface remained active, it typically reasserted the correct ARP mapping, limiting persistence of the inconsistent state.

## 6.3 DAI Dependency on DHCP Snooping

Dynamic ARP Inspection (DAI) was evaluated as a mechanism for enforcing data-plane integrity by validating ARP packets against a trusted binding source.

DAI operates by inspecting ARP requests and replies on untrusted interfaces and validating the IP-to-MAC mapping against an authoritative reference. In standard enterprise deployments, this reference is the DHCP Snooping binding table.

When an ARP packet is received on an untrusted port, DAI performs the following validation steps:

- Extracts the sender IP and MAC address from the ARP payload.

- Compares the IP–MAC pair against the DHCP Snooping binding table.

- Drops the packet if no matching binding exists or if the mapping is inconsistent.

This mechanism ensures that endpoints cannot claim ownership of IP addresses that were not legitimately assigned to them.

### 6.3.1 Validation Dependency and Static Addressing Implications

During testing, it was observed that DAI enforcement depends entirely on the availability of a valid DHCP Snooping binding entry.

In this laboratory topology:

  - The default gateway IP address (192.168.10.1) was statically configured on the router.

  - The gateway IP was not obtained via DHCP.

  - No corresponding entry existed in the DHCP Snooping binding table.

As a result:

- DAI had no dynamic reference entry for validating ARP packets claiming ownership of 192.168.10.1.

- ARP validation for statically addressed infrastructure required explicit configuration.

This reveals an important architectural dependency:

DAI derives its validation authority from DHCP Snooping unless explicitly supplemented with static bindings or ARP ACL entries.

In environments where infrastructure devices (e.g., default gateways, servers, printers) are statically addressed:

- DHCP Snooping will not automatically generate binding entries.

- DAI will lack dynamic validation context for those IP addresses.

- Additional configuration is required to ensure deterministic ARP validation.

This scenario is common in enterprise deployments and must be accounted for during security design.

### 6.3.2 Production-Grade Mitigation Approaches

In enterprise environments, two primary methods are used to ensure DAI validation for statically addressed devices:

#### Method 1 — Static DHCP Snooping Binding Entry

ip dhcp snooping binding <MAC> vlan <VLAN> <IP> interface <PORT>

This command manually inserts a trusted binding entry, allowing DAI and IP Source Guard to validate ARP packets against explicitly defined infrastructure mappings.

#### Method 2 — ARP Access Control List (ARP ACL)

arp access-list STATIC-GW
permit ip host <IP> mac host <MAC>
ip arp inspection filter STATIC-GW vlan <VLAN>

This approach creates a static ARP validation rule independent of DHCP Snooping.

In addition to DAI, IP Source Guard further enforces binding integrity by restricting Layer 3 traffic to DHCP Snooping-derived IP–MAC associations at the ingress interface. This ensures that even if ARP manipulation occurs, traffic sourced from unauthorized IP–MAC combinations is blocked at the access port.

### 6.3.3 Tooling Limitation — Packet Tracer

Packet Tracer does not fully support dynamic ARP injection testing or consistent static binding manipulation for infrastructure devices. As a result:

- Complete MITM-style ARP spoofing scenarios could not be fully reproduced.

- Validation behavior was analyzed through duplicate-IP simulation and architectural dependency evaluation.

Despite these limitations, the dependency between DAI and DHCP Snooping binding integrity was successfully understood.

# 7. Layered Defense Model Validation Results

No single control provides complete Layer 2 security. Effective enterprise protection requires coordinated enforcement across trust boundaries, identity validation, rate control, and data-plane integrity mechanisms.

This laboratory validated that Layer 2 security is not achieved through isolated feature activation, but through deliberate control chaining and dependency alignment.

The following control relationships were confirmed:

- DHCP Snooping defines explicit server-side trust boundaries and establishes the authoritative binding source.

- Rate Limiting constrains packet-velocity–based starvation attempts.

- Port Security enforces per-interface identity constraints and mitigates MAC rotation abuse.

- Dynamic ARP Inspection (DAI) protects data-plane integrity through binding validation.

- Static Bindings and ARP ACLs close validation gaps for statically addressed infrastructure.

- IP Source Guard enforces Layer 3 ingress integrity using the same authoritative binding source.

Testing demonstrated that control interaction is as critical as control configuration. Misalignment of trust boundaries, improper Option 82 handling, or missing static validation entries can undermine otherwise correctly deployed mechanisms.

The validated architecture reflects real-world enterprise design principles:

Security controls must operate cooperatively.
Trust must be explicit.
Validation must be chained.
Dependencies must be understood.

Layer 2 security is therefore not a feature-based configuration exercise, but an architectural discipline.

| Control Mechanism | Attack Type Mitigated | Dependency | Residual Risk |
| --- | --- | --- | --- |
| DHCP Snooping | Rogue DHCP Server, DHCP Spoofing | None (Root Trust Anchor) | Starvation via MAC rotation |
| DHCP Rate Limiting | DHCP Starvation (High-rate) | DHCP Snooping | Low-rate MAC rotation |
| Port Security | MAC Rotation, Identity Abuse | None | Insider misuse within limit |
| DAI | ARP Spoofing, Gateway Impersonation | DHCP Snooping Binding Table | Static IP validation gap |
| Static Binding / ARP ACL | Gateway Spoofing (Static IP) | Manual Configuration | Human misconfiguration |
| IP Source Guard | IP Spoofing | DHCP Snooping Binding Table | Static addressing dependency |